Scalable Hybrid Quantum-Classical Optimization for Oil Reservoir Well Placement Using QUBO and QAOA

# ✍️ Abstract

This paper presents a hybrid quantum-classical framework for optimizing oil reservoir well placement using Quadratic Unconstrained Binary Optimization (QUBO) and the Quantum Approximate Optimization Algorithm (QAOA). The well placement problem is formulated as a QUBO model incorporating production maximization and interference minimization. Classical optimization techniques, including exact enumeration and simulated annealing, are compared against quantum-inspired and quantum-based approaches such as QAOA. Experimental results demonstrate that QAOA achieves solution quality comparable to classical methods for small-scale instances (6–10 wells), while incurring higher computational cost due to variational optimization overhead. A scalability analysis reveals the limitations of classical exact methods and highlights the potential of quantum algorithms for larger problem sizes. The proposed framework provides a foundation for integrating quantum optimization into real-world petroleum engineering applications.

# 📚 1. Introduction

Optimizing well placement in oil reservoirs is a critical problem in petroleum engineering, directly impacting production efficiency and economic performance. The problem is inherently combinatorial and can be classified as a non-deterministic polynomial-time hard (NP-hard) optimization problem due to the exponential growth of possible configurations with the number of candidate wells.

Traditional approaches rely on heuristic algorithms, simulation-based optimization, or gradient-based methods. While effective for moderate-scale problems, these methods often struggle with scalability and global optimality.

Recent advances in quantum computing have introduced new paradigms for solving combinatorial optimization problems. In particular, the Quantum Approximate Optimization Algorithm (QAOA) has emerged as a promising hybrid quantum-classical algorithm capable of solving QUBO-formulated problems.

This work proposes a hybrid framework that:

- Formulates well placement as a QUBO problem

- Incorporates reservoir physics (production and interference)

- Compares classical and quantum solvers

- Evaluates performance across multiple metrics

- Investigates scalability behavior

# ⚙️ 2. Problem Formulation

The well placement problem is formulated as a QUBO:

min⁡x∈{0,1}nxTQx\min_{x \in \{0,1\}^n} x^T Q xminx∈{0,1}n​xTQx

Where:

- xi∈{0,1}x_i \in \{0,1\}xi​∈{0,1}: decision variable (well selected or not)

- QQQ: cost matrix encoding production and interference

## Objective Components:

### 1. Production Maximization

Maximize oil production potential of selected wells

### 2. Interference Penalty

Minimize negative interactions between nearby wells

Thus, the QUBO matrix is constructed as:

Q=−P+λIQ = -P + \lambda IQ=−P+λI

Where:

- PPP: production matrix

- III: interference matrix

- λ\lambdaλ: penalty weight

# 🧪 3. Methodology

## 3.1 Data Generation

Synthetic reservoir data is generated including:

- Porosity

- Permeability

- Pressure

- Distance matrix

## 3.2 QUBO Construction

The QUBOBuilder constructs the matrix:

- Diagonal terms → production

- Off-diagonal → interference penalties

## 3.3 Classical Solvers

### Exact Solver

- Enumerates all 2n2^n2n combinations

- Guarantees optimal solution

### Simulated Annealing (SA)

- Heuristic optimization

- Fast but approximate

## 3.4 Quantum Solver (QAOA)

QAOA is used to solve the QUBO:

- Uses parameterized quantum circuits

- Optimized via classical optimizer (COBYLA)

- Implemented using Qiskit

# ⚙️ 4. Experimental Setup

| Parameter | Value |
| --- | --- |
| Wells | 6, 8, 10 |
| Runs | 10 |
| QAOA Layers (p) | 2 |
| Optimizer | COBYLA |

# 📊 5. Results

## 5.1 Objective Energy

All methods achieved comparable energy values across problem sizes.

👉 Interpretation:
The QUBO formulation is consistent and solver-independent.

## 5.2 Production

Production values were nearly identical across all solvers.

👉 Indicates:

- Stable optimization landscape

- No solver bias

## 5.3 Interference

All methods maintained similar interference penalties.

## 5.4 Runtime Analysis

Significant differences observed:

- Exact → fastest (small n only)

- SA → moderate

- QAOA → slowest

## 🔥 5.5 Optimality Gap (NEW CONTRIBUTION)

Defined as:

Gap=Emethod−Eoptimal\text{Gap} = E_{method} - E_{optimal}Gap=Emethod​−Eoptimal​

Observation:

- Gap ≈ 0 for all methods

- QAOA matches optimal solution quality

# 📈 6. Scalability Analysis

Complexity comparison:

### Exact Solver:

O(2n)O(2^n)O(2n)

### Simulated Annealing:

O(n⋅k)O(n \cdot k)O(n⋅k)

### QAOA:

O(p⋅Ccircuit)O(p \cdot C_{circuit})O(p⋅Ccircuit​)

👉 Key Insight:

- Exact becomes infeasible as n↑n \uparrown↑

- QAOA scales better theoretically

# 🧠 7. Discussion

The results indicate that:

- QAOA achieves optimal-quality solutions

- Classical solvers outperform QAOA in runtime for small problems

- Quantum advantage is not observable at small scale

- Potential advantage emerges for larger systems

# 🏁 8. Conclusion

This study introduced a hybrid quantum-classical framework for oil reservoir well placement optimization using QUBO and QAOA.

Key findings:

- QAOA matches classical solution quality

- Runtime overhead remains a challenge

- Scalability analysis supports future quantum advantage

Future work includes:

- Testing on real reservoir datasets

- Increasing QAOA depth

- Deployment on real quantum hardware

# 📌 Keywords

QUBO, QAOA, Oil Reservoir Optimization, Quantum Computing, Well Placement, Hybrid Optimization